OCT-Embedding and Flash-Freezing Protocol

Solutions needed

Dry ice 							2-3 lbs

Optimal Cutting Temperature (OCT) compound 	as needed

Materials needed

Falcon tube

Silicon ice cube tray/mold

Metal forceps (sharp or blunt)

Red plastic tub & lid

Large polystyrene cooler

Paper towel

Protocol

- Bring the red plastic tub and lid to chem stores to pick up dry ice.

  - Chem stores is open from 9-11:30am and 12:30-4pm Monday through Friday.

- Empty dry ice from red tub into large polystyrene cooler then shake the cooler to generate a flat surface for the silicone ice cube tray/mold to sit atop and remain level and undistorted.

- Pour a small amount of OCT to one silicone ice cube mold, only use enough to fully coat the bottom of the mold.

- Transfer the silicone tray/mold into the polystyrene cooler.

  - Make sure the tray and the liquid within it are sitting level atop the dry ice.

  - Replace the lid and wait ~5-10 minutes for OCT to freeze (OCT will turn white once it is frozen).

- Weigh the brain just prior to starting step 6.

** The remaining steps must be performed quickly to prevent the OCT from thawing. **

- Remove the silicone tray/mold from the polystyrene cooler.

  - Add OCT to mold containing frozen base layer, fill to approximately 2/3 full and avoid producing air bubbles.

  - Use a metal forceps to remove and/or move any bubbles away to the side.

- Place the brain into the OCT.

  - Use metal forceps to move the brain to the center of the mold (diagonal with the caudal side up) See photo 1 for how to orient the brain. Remove and/or move any bubbles away to the side.

  - Pour OCT on top of brain, ensuring that it is fully submerged and covered in OCT.

  - Take note of the orientation of the brain and which corner the dorsal side is on.

- Put the ice cube tray back into the polystyrene cooler for ~5-10 minutes, until block is completely frozen.

- Remove the tray. Using a sharpie, mark the corner where the dorsal side of the brain is facing.

- Remove the OCT-embedded brain from the silicone ice cube tray/mold by everting the mold into an empty falcon tube.

- Place the falcon tube containing the OCT-embedded brain into the -80°C freezer.

Dorsal

Photo 1.

Top view – caudal side shown

Right hemisphere

Left hemisphere

Ventral